EE-380L: Communication System Lab – Project

Instructors: Zartash Afzal Uzmi; Naveed-ul-Hassan

Lab Engineer: Irfan Ullah

Teaching Assistants: Usman Nazir, Humera Kaleem, Saad Sadat Cheema, Usman Rauf

Instructions:

- Carefully read each and every point that is written below

- Project has to be done in a group of two

- Proposal submission: 6 April 2016.

- Due Date: Friday 29 April 2016.

- Viva: Monday 2nd May 2016.

Objectives:

- Design and test an FM transceiver.

- To develop practical expertise in Software Defined Radios (the new emerging field in communication).

- To develop some intuition and practical experience in the field of radio frequency design and measurement.

- To demonstrate a simple oscillator by use of a bipolar junction transistor (BJT) and a tank circuit.

- Reception of the transmitted frequency modulated (FM) voice signal by an SDR receiver.

Tasks:

- Transmitter:

- Implement the Colpitts oscillator by using a BJT as shown in the schematics below

- Place an electric microphone between Port A and Port B

- Leave a wire hanging at the output. This will act as an antenna

- Place an FM receiver at some distance from the antenna and hear the signal being picked up by the microphone

- Understand the working of the circuit

Note: You can change the oscillation frequency by modifying the values of inductor!

ii) 	Receiver

- Implement software defined radio receiver using Matlab and RTL-SDR hardware.

- Design FM discriminator receiver in Matlab Simulink and interface it with RTL-SDR receiver block.

- And set a specific channel frequency of your receiver(the freuecy at which you are transmitting signal) to listen to your transmitted signal from your transmitter designed on PCB.

- Analyze the received spectrum using spectrum analyzer block(simulink) and also analyze time domain signal using time scope.

Methodology: (Implement FM discriminator receiver)

    - Open Simulink and create a new model window save it with fm_receiver.slx.

    - Take RTL-SDR Receiver block from Simulink library.

    - Filter the output samples of RTL-SDR Receiver block using FIR filter with decimation factor of 10 and allow multi rate processing.

    - Let x[n] be the output of now use blocks to do the following operation on x[n].

Yn=angle(conjugate(Xn)*X[n-1])

Where x*[n] is the conjugate of x[n].

    - Again filter the output Y[n] with FIR filter with decimation factor of 5.

    - Now take the FM De-emphasis Filter and set the filter region to Europe + Asia and set the sampling frequency to 48 KHz.

    - Now connect a speaker (To Audio Device block) and Spectrum

Analyzer block to the output of FM De-emphasis Filter.

Learning Perspective:

After doing this assignment, you should be able to answer the following questions:

- What is a tank circuit and how it works? Why it is used here? Locate it in the schematics given?

- How the frequency modulation is being done? And which part is responsible for doing FM in the circuit?

- How the oscillation frequency is calculated? And is dependent on which components? Locate!

- What is the role of different components which can be seen in the given schematics?

- Can an inductor be made if you are given with a wire? If yes, then how its’ inductance is calculated?

Announcements:

- Grading policy and Submission policy will be announced subsequently.